Design approach:

First, I want to think about the system and how I can run this device. I go with a module base concept. This module can cut-off the system when it detects over voltage and over current. And module run by a MCU. Because by MCU we can read the voltage through it’s ADC pin.

Implementation Process:

Input line voltage converted high voltage DC through rectifier and filter capacitor. This HVDC is converted 12VDC by a non-isolated buck IC (OB222NCP). To convert 12VDC to 5VDC I use RY8310 buck IC. And to convert the 5VDC to 3.3VDC I use RY6212 IC.

I use 3.3VDC voltage reference for the MCU.

Simple voltage divider is used for reading ADC through MCU.

There is many way to detect surge current. I use current transformer and use a offset voltage system to measure current so that it never goes below zero. In the simulation, I use hall sensor because there is no CT coil library.

Result:

This device can measure mili seconds surge properly. Because relay switching speed works within mili seconds range. And user can set their limit also.

Note: This device is non-isolated. So. Optocoupler is used for user to interface with the device to set the value.